Texas Instruments OPT8241 CDK

Assembly Instructions

Texas Instruments

Date: 10/28/2015

Version: 1.1

D3 Engineering, LLC

1057 E. Henrietta Rd

Rochester, NY  14623

p. 585.429.1550

f. 585.429.1551

www.d3engineering.com

- Table of Contents

1	Purpose	3

2	Paint	3

3	Diffuser mount assembly ASY-0005	3

4	Illumination Board assembly ASY-0006	3

5	Sensor Board assembly ASY-0007	3

6	Stack Assembly for focus and alignment	3

7	Program Firmware	4

8	Focusing	4

9	Alignment	4

10	Final Assembly ASY-0008	4

11	Testing	4

12	Packaging	4

13	Reference BOMs	5

13.1	Diffuser Assembly ASY-0005	5

13.2	Illumination board Assembly ASY-0006	5

13.3	Sensor board Assembly ASY-0007	5

13.4	OPT8241 board stack ASY-0008	6

13.5	OPT8241 CDK	6

14	Revision History	6

# Purpose

This document is intended to present the instructions to assemble a complete Texas Instruments OPT8241 CDK unit.

Aesthetics is paramount.

# Paint

- Paint FAB-0019 and FAB-0020 flat black and let dry.

# Diffuser mount assembly ASY-0005

- With FAB-0019 diffuser mount bottom side up place a very, very, small amount of MSC-0019 (NOA-87) adhesive on the back side of the top aperture of FAB-0019 using a 27G dispensing needle.

- Install OPD-0008 diffuser glass into FAB-0019 diffuser mount such that the smooth undiffused side will face out the aperture of FAB-0019.

- Cure MSC-0019 (NOA-87) adhesive with UV wand per manufactures recommendation.

# Illumination Board assembly ASY-0006

- Place PN ASY-0005 diffuser mount assembly on laser diodes D11, D12, D13 and D16.

- Place PN FAB-0020 diffuser retainer on to PN ASY-0005 diffuser mounts.

- Secure PN FAB-0020 diffuser retainer with PN HRW-0020-010 screw and PN HRW-0007-ZN nut to illumination board using Loctite 243.

- Secure PN ASY-0005 diffuser mount assemblies with a fillet of NEA 123 to the laser diodes D11, D12, D13, D16 and the circuit board in 2 places per diffuser mount.

- Cure locations with UV wand per manufacturer’s recommendation.

# Sensor Board assembly ASY-0007

- Clean sensor U3 on the sensor board with Acetone and a lens cleaning swab.

- Install PN FAB-0021 Lens mount to the sensor board with PN HRW-0006-012 screw and PN HRW-0022-ZN nut. Make sure that the mount is snug to the board but will still allow some lateral movement for alignment in section 9 Alignment.

- Install PN FAB-0022 Tripod mount to bottom of sensor board with PN HRW-0023-008 screws using Loctite 243.

- Install PN OPD-0010 Lens into PN FAB-0021 Lens mount.

# Stack Assembly for focus and alignment

- Assemble Illumination board to Sensor board with PN HRW-0023-016 screw, PN HRW-0021-010 spacer and PN HRW-0014-ZN nut in only 2 places diagonal from each other in order to allow of easy disassembly in section 10 Final Assembly.

- Install PN CBL-0008 cable between J1 on the illumination board and J1 on the sensor board, such that the red stripe on the cable is towards the top edge of the boards.

# Program Firmware

- Refer to CDK-OPT8241 -FX2 firmware upgrade document.

# Focusing

- Setup board on tripod 1m away from 1951 USAF resolution target printed on 11x17 paper.

- Apply power to board and connect USB to PC.

- Start Voxel Viewer.

- Right Click the point cloud and select to view the Amplitude image.

- Use the amplitude image to focus on the checkerboard.

# Alignment

- Setup board on tripod 1m away from1951 USAF resolution target printed on 11x17 paper.

- Apply power to board and connect USB to PC.

- Start Voxel Viewer.

- Right Click the point cloud and select to view the Amplitude image.

- Use the amplitude image to align the lens mount.

- Adjust PN FAB-0021 lens mount such that the image is centered.

- Carefully apply Loctite 222 thread locker to PN HRW-0006-012 screw and/or PN HRW-0022-ZN nut and tighten.

- Verify that the image is still centered and the lens is still in focus, readjust if needed.

# Final Assembly ASY-0008

- Remove Illumination board from Sensor board.

- Seal around the perimeter of PN FAB-0021 Lens mount by applying Loctite 480 instant adhesive.

- Cure Loctite 480 using Loctite 7452 ensuring that the lens remains free from contaminants.

- Apply three small fillets of black RTV between PN OPD-0010 Lens and PN FAB-0021Lens mount, care shall be taken not to drastically affect the threads on the lens.

- Fold cable CBL-0008 such that it tucks in between both boards.

- Reinstall Illumination board to Sensor board with PN HRW-0023-016 screw, PN HRW-0021-010 spacer, PN 3122 nylon washer and PN HRW-0014-ZN nut in 4 places using Loctite 222

# Testing

- Test per Test Documents

# Packaging

- Place tested board stack in PKG-0002 static bag.

- A fix LBL-0004 to Static bag.

# Reference BOMs

## Diffuser Assembly ASY-0005

| Item Number | Part Number | Description | Unit of Measure | QTY |
| --- | --- | --- | --- | --- |
| 0001 | FAB-0019 | Diffuser Mount | each | 1 |
| 0003 | MSC-0019 | NOA-87 | as needed | 0.0001 |
| 0002 | OPD-0009 | Diffuser | each | 1 |

## Illumination board Assembly ASY-0006

| Item Number | Part Number | Description | Unit of Measure | QTY |
| --- | --- | --- | --- | --- |
| 0001 | OPT8241-IB | Tin Tin Illumination Board | each | 1 |
| 0002 | ASY-0005 | Tin Tin Diffuser Assembly | each | 4 |
| 0003 | FAB-0020 | Retainer, Diffuser Mount | each | 1 |
| 0004 | HRW-0020-010 | Screw, SHC, M2, Black Oxide, 10MM | each | 1 |
| 0005 | MSC-0017 | Thread Lock, Medium strength | as needed | 1 |
| 0006 | HRW-0007-ZN | Nut, Hex, M2, Zn pltd steel | each | 1 |
| 0007 | MSC-0020 | NEA-123 | as needed | 4 |

## Sensor board Assembly ASY-0007

| Item Number | Part Number | Description | Unit of Measure | QTY |
| --- | --- | --- | --- | --- |
| 0001 | OPT8241-SB | Tin Tin Sensor Board | each | 1 |
| 0002 | FAB-0021 | Mount, Lens, M10 | each | 1 |
| 0003 | HRW-0006-012 | Screw, SHC, M1.6, Black Oxide, 12MM | each | 2 |
| 0004 | HRW-0022-ZN | Nut, Hex, M1.6, Zn pltd Steel | each | 2 |
| 0005 | MSC-0017 | Thread Lock, Medium strength | as needed | 1 |
| 0006 | OPD-0010 | Lens, M10 | each | 1 |
| 0007 | FAB-0022 | Mount, Board to Tripod | each | 1 |
| 0008 | HRW-0023-008 | Screw, SHC, M3, Black Oxide, 8MM | each | 2 |
| 0009 | 3122 | Keystone Electronics nylon washer | each | 4 |

## OPT8241 board stack ASY-0008

| Item Number | Part Number | Description | Unit of Measure | QTY |
| --- | --- | --- | --- | --- |
| 0001 | ASY-0007 | Tin Tin Sensor board Assembly | each | 1 |
| 0002 | ASY-0006 | Tin Tin Illumination board Assembly | each | 1 |
| 0003 | CBL-0008 | Cable, Ribbon, 24 conductor, 2" | each | 1 |
| 0004 | HRW-0021-010 | Spacer, 4.5 Rnd, M3 Clearance, 10MM | each | 4 |
| 0005 | HRW-0023-016 | Screw, SHC, M3, Black Oxide, 16MM | each | 4 |
| 0006 | HRW-0014-ZN | Nut, Hex, M3, Zn pltd Steel | each | 4 |
| 0007 | MSC-0017 | Thread Lock, Medium strength | as needed | 1 |

## OPT8241 CDK

| Item Number | Part Number | Description | Unit of Measure | QTY |
| --- | --- | --- | --- | --- |
| 0001 | ASY-0008 | Tin Tin Board Stack | each | 1 |
| 0002 | PKG-0002 | Tin Tin Static Bag 4'x6' | each | 1 |
| 0003 | LBL-0004 | Label, Laser | each | 1 |

# Revision History

Rev 0.1 Pre-Release 1

Rev 0.2 Pre-Release 2 post pilot build

Rev 1.0 Production release

Rev 1.1 Cleaned up release